UNIVERSITY OF WOLLONGONG IN DUBAI

Accessibility Scene
Hazard Assistant

An Integrated Computer Vision Application for Accessible Scene Guidance

GITHUB REPO - https://github.com/NXRJ/CSIT435-Project

CSCI435 - Computer Vision Algorithms and Systems
Spring 2026

Mehdi Leghmizi  |  8528834

Neeraj Santosh  |  8329345

Muhammad Soban  |  8555588

Zachary Bracke  |  8947405

Mostafa Shalash  |  7391493

Luis Vargas | 7655538

Lecturer: Dr Patrick Mukala
Report date: 27 June 2026

# Report at a glance

We created and developed a local deployable web application that combines the following: image enhancement, edge detection, object proposal, custom-trained object recognition, moving-object detection, and object tracking all in one workflow. The system should accept an uploaded image or a webcam capture or video, returning annotated visual results, text guidance, latency, and estimated throughput

Outcome: the notebook measured 93% avg precision, 94.6% recall, and 93.1% F1 across seven different scene conditions. The full still-frame pipeline managed to average 50.1 FPS (20ms), which exceeded the 10 FPS requirement

## Abstract

## This project looks into how multiple computer vision techniques could operate together as a whole usable system rather than singular isolated lab exercises. Our concept is an accessible scene assistant used to recognize three marker types: stop, warning, and safe route. A custom dataset of 560 marker crops is used and generated in the notebook. HOG, HSV, Intensity and Hu moment are used to compare an RBF support vector machine, random forest and k-nearest neighbor classifier. Then a selected SVM is integrated with color proposals, adaptive Cany edges, CLAHE enhancement, MOG2 background modelling and centroid tracking. Evaluation includes held-out recognition, class-aware scene detection, lighting and orientation changes, noise, blur, occlusion, clutter, blank inputs and CPU speed. Results were very strong in the controlled synthetic environment, but we also identify gaps between it and real life and the need for user testing before any real deployment.

## Submission components

Executed Jupyter notebook: CSCI435_Project.ipynb.

Dependency file and reproducible execution instructions in README.md.

Two-minute video demonstration from actual pipeline outputs.

Editable DOCX report and submission-ready PDF report.

Timed defence guide, robust sample inputs and requirement-to-evidence matrix.

# Introduction

## Problem statement

People with impaired or below optimal vision might need a quick indication of any warnings, route cues, visible boundaries and movement in a camera view. A lot of the classroom demonstrations usually process one image with only one algorithm, and that doesn’t show how an actual usable application should validate input, combine results or communicate any uncertainty. Taking this into account, our goal was to create a single interface that links low-level, mid-level and high-level vision tasks around a clear accessibility user story.

## User story

User story. As a user, I want to upload an image or video, or capture a webcam image, so that one interface enhances the scene, recognises marker-like objects, highlights boundaries and movement, and gives concise guidance.

## Selected vision capabilities

Image enhancement: gray-world colour balancing followed by CLAHE on the luminance channel.

Edge detection: adaptive Canny thresholds with morphological closing and a visible edge overlay.

Object detection: HSV colour segmentation, morphology, contour filtering and bounding boxes.

Object recognition: an RBF SVM trained on the group's generated custom marker dataset.

Change and moving-object detection: MOG2 adaptive background modelling on video frames.

Object tracking: centroid association with persistent IDs and missing-frame tolerance.

## Scope and safety

The application is an academic proof-of-concept, not a certified safety device. Its vocabulary and synthetic evaluation are deliberately restricted so that every training and testing step is reproducible. Real world use would require representative data, calibrated depth, accessibility studies, privacy review and a much more conservative false-negative analysis to prevent use and legal issues.

# System architecture

Figure 1. System architecture and data flow.

The front-end and back-end are both packaged in the notebook. Gradio gives us the browser interface while Python, OpenCV and scikit-learn provide the processing and model layers. An input is validated and resized to a max width of 640 pixels. Enhanced frames provides edge and color-proposal branches. Candidate pixels are classified using the custom SVM. Video frames go through MOG2 and the centroid tracker. Fusion draws boxes, labels and track IDs, builds a text guidance sentence and reports the resulted latency/FPS.

## Frontend and input modalities

The Image / Webcam tab accepts an upload or camera capture, then shows four outputs: the integrated annotated result, enhanced image, edge view and guidance. The Video tab would also accept an uploaded file, applies the same frame pipeline plus motion/tracking, and returns an annotated video and aggregate metrics. These two paths achieve the requirement of at least two input modalities while keeping the interface consistent.

Figure 2. Verified local Gradio interface in the Image / Webcam mode.

# Implementation details

## Enhancement and edge processing

Gray-world balancing estimates a target mean across BGR channels and limits channel scaling to avoid extreme colour shifts. The result is then converted to CIELAB, where CLAHE will be applied only to luminance. Adaptive Canny thresholds are then calculated from the median of a Gaussian-smoothed grayscale image, a 3 x 3 closing operation joins short breaks. This branch supplies a clear boundary view but does not directly claim actual semantic meaning.

## Object proposal and custom recognition

HSV thresholds form red, yellow and green proposal masks. Opening removes isolated pixels and closing joins small gaps. Original and enhanced pixels are both searched using separate colour masks so a dominant single colour cannot be lost during balancing and adjacent marker colours cannot merge. Contours below 300 pixels or outside broad aspect-ratio limits are rejected. Each remaining object is centred on a neutral 96 x 96 canvas at a fixed foreground scale. This canonicalisation solved an important training/inference mismatch: the recogniser now sees the same object scale during both phases. Proposal masks are grouped by the SVM using pre-enhancement validated pixels to preserve the training appearance distribution. When the synthetic crop model is unsure, a conservative fusion rule checks real evidence: red octagonal shapes with a bright symbol support stop, yellow triangular shapes support warning, and green circular shapes with a bright internal symbol support safe. This recovered examples without replacing the learned model.

enhanced = enhance_image(frame)
edges = adaptive_canny(enhanced)
detections = detect_markers(
    enhanced, self.model, classification_frame=frame
)

## Motion detection and tracking

For video, OpenCV's MOG2 model learns an adaptive background. Shadow-valued pixels are removed by a high binary threshold, followed by opening and dilation. After four warm up frames, contours between 500 pixels and 65% of the frame area become moving regions. A lightweight centroid tracker links each current box with the nearest existing centre, assigns a new ID when required and removes IDs after eight missed frames. This method is fast and easy to explain, although its drawback is it can exchange identities when objects cross.

## Input validation and robustness controls

•  Rejects empty, incorrect, invalid input images with readable error messages.

•  Supports grayscale and four-channel images by converting them to three-channel BGR.

•  Cuts down wide images maintaining aspect ratio to limit latency and memory usage.

•  Provides a no-marker message for empty or not matching images.

•  Outputs video file in MP4 format with AVI fallback if the local codec is not initialized.

# Custom data and model training

## Dataset construction

The notebook generates 560 labelled images, with 140 examples for each of the four classes: stop, warning, safe and other. Variations include background level gradient, scale, rotation, brightness, blur, Gaussian noise and partial occlusion. The other class is a red square with a diagonal line, intentionally designed to pass the colour proposal phase and force the recognizer to use shape evidence. Random seeds are fixed for reproducibility.

Figure 3. Examples from the custom synthetic dataset.

## Features and candidate models

Each canonical crop is represented by HOG gradients, 18-bin hue and 12-bin saturation histograms, four intensity/colour statistics and seven log-transformed Hu moments. We used a stratified 75/25 split and compared three lightweight classifiers on the same samples. Scaling is included inside the SVM and k-NN pipelines to avoid information leakage from the held-out split.

| Model | Validation accuracy | Training (s) | Inference (ms/crop) |
| --- | --- | --- | --- |
| SVM (RBF) | 100.0% | 0.105 | 0.070 |
| Random forest | 100.0% | 0.168 | 0.193 |
| k-NN | 100.0% | 0.005 | 9.172 |

Table 1. Candidate-model comparison from the executed notebook.

## Model choice

All three models separated the controlled crop dataset perfectly, so crop accuracy alone could not justify a choice. We selected the RBF SVM because it combined a non-linear boundary, class balancing, probability output for user-facing confidence and low per-crop inference time. The trained pipeline is saved as a joblib artifact and is also recreated from source whenever the notebook is run.

Figure 4. Held-out confusion matrix for the selected SVM.

Interpretation warning. The 100% held-out crop accuracy applies only to the generated marker distribution. It must not be presented as real-world safety accuracy. End-to-end scene metrics are more demanding and are reported separately.

# Experiments and results

## Evaluation protocol

Recognition robustness used 35 unseen crops per class and condition. End-to-end detection used 12 scenes per condition, with one to three target classes per scene and seeds not used for training. A prediction counted as a true positive only when its class matched and its intersection-over-union with an unused ground-truth box was at least 0.30. Precision, recall, F1 and mean matched IoU were then aggregated. The seven scene conditions were bright, dim, rotated, blurred, noisy, occluded and cluttered.

| Condition | Precision | Recall | F1 | Mean IoU | Recognition accuracy |
| --- | --- | --- | --- | --- | --- |
| Bright | 100.0% | 100.0% | 100.0% | 0.765 | 100.0% |
| Dim | 95.5% | 87.5% | 91.3% | 0.727 | 100.0% |
| Rotated | 100.0% | 100.0% | 100.0% | 0.770 | 94.3% |
| Blurred | 100.0% | 100.0% | 100.0% | 0.803 | 100.0% |
| Noisy | 100.0% | 100.0% | 100.0% | 0.752 | 100.0% |
| Occluded | 95.0% | 79.2% | 86.4% | 0.724 | 100.0% |
| Cluttered | 60.5% | 95.8% | 74.2% | 0.750 | N/A |

Table 2. Condition-wise robustness results from the executed notebook.

Across the seven conditions, mean precision was 93.0%, mean recall was 94.6% and mean F1 was 93.1%. Blurred and noisy generated scenes reached perfect detection scores in this run. Rotated, occluded and cluttered scenes were harder. Clutter reduced precision because saturated distractors sometimes survived proposal filtering; rotated and occluded targets mainly reduced recall. This pattern is consistent with the proposal/classification design.

Figure 5. Detection and recognition robustness by condition.

## Performance

| Measure | Result |
| --- | --- |
| Still-frame mean latency | 19.96 ms |
| Still-frame median latency | 19.70 ms |
| Still-frame 95th-percentile latency | 21.74 ms |
| Estimated still-frame throughput | 50.09 FPS |
| Processed-video mean latency | 16.75 ms/frame |
| Processed-video estimated throughput | 59.72 FPS |
| Rubric threshold | 10 FPS - met |

Table 3. CPU timing measured over the complete integrated pipeline.

The benchmark includes validation, resizing, enhancement, edges, proposal generation, feature extraction, SVM inference, drawing and guidance. Video timing also includes MOG2 and tracking. FPS is computed as the reciprocal of processing latency and therefore measures processing capacity rather than camera-display refresh rate. Results depend on hardware, but the tested run had a substantial margin above the 10 FPS criterion.

## Qualitative results

Figure 6. End-to-end image workflow and generated guidance.

The clean defence scene demonstrates how the tasks support one another. Enhancement normalises contrast, the edge view exposes structural boundaries, colour/contour proposals localise candidates, the SVM labels all three markers and the fusion layer produces: 'Safe-route marker on the right; warning marker in the centre; stop marker on the left.' Video examples add motion boxes and persistent IDs using the same base pipeline.

## Correctness and edge-case tests

•  Blank scene of all markers: zero marker detection and no markers legible message.

•  Small 12 x 12 pixel scene: value error thrown and not run through the OpenCV function.

•  Dark, rotated, blurry, noisy and partially occluded scenes: processed and scored based on their conditions.

•  Cluttered scene: false positives are counted and not filtered out of the results.

•  User-provided real style regression scene: stop detected at 98.0% and safe detected at 94.6%.

•  Generated 90 frame video: processed successfully with motion/tracking outputs.

# Challenges and critical analysis

## Training-to-inference mismatch

An early version classified padded scene crops directly even though training used tightly framed 96 x 96 examples. Crop accuracy looked perfect, but scene recall was only 7.1%. This was a useful warning against reporting a convenient metric without testing the complete system. Canonical foreground scaling and recognition from pre-enhancement pixels raised the verified mean scene F1 to the value reported in Table 2.

## Remaining limitations

Synthetic-to-real domain gap: natural signs, reflections, weathering and camera compression are not represented.

Colour dependence: extreme colour casts or desaturation may prevent proposal generation before the SVM runs.

No calibrated distance: left/centre/right guidance is relative image position, not metric depth or collision risk.

Motion is not semantic: MOG2 identifies changing regions but cannot state whether they are people or vehicles.

Simple tracking: nearest-centroid matching may exchange IDs during crossing or prolonged occlusion.

Safety and accessibility: no target-user study, audio/haptic interface or formal false-negative safety threshold was completed.

## UI and deployment trade-offs

Gradio made the application fast to reproduce and suitable for a live local defence. It also exposes upload and webcam sources without maintaining a separate JavaScript codebase. The trade-off is less control over mobile layout and browser codec behaviour than a custom frontend. Local hosting avoids sending camera data to a cloud service, but the prototype does not yet implement persistent privacy controls or authentication.

# Individual contributions

The following table records the agreed primary workstreams. Integration, review and defence preparation were shared so that each member contributes to coding, documentation and presentation as required.

| Member | Student ID | Primary coding | Documentation | Defence role |
| --- | --- | --- | --- | --- |
| Mehdi Leghmizi | 8528834 | UI integration and input handling | User story and requirements | Opening and UI demo |
| Neeraj Santosh | 8329345 | Custom dataset and model training | Training methodology | Model selection |
| Muhammad Soban | 8555588 | Enhancement, edges and robustness tests | Experiment methodology | Image robustness |
| Zachary Bracke | 8947405 | Motion, tracking and performance | Results and critical analysis | Video and performance |
| Mostafa Shalash | 7391493 | System integration and QA | Architecture, conclusion and references | Architecture and closing |
| Luis Vargas | 7655538 | Feature engineering & viz | Reproduction guide & appendix | Features & evaluation |

Table 4. Group contribution record and defence responsibilities.

# Conclusion and future work

The project demonstrates a working integrated system rather than a collection of isolated scripts. Six permissible computer vision capabilities operate inside one image/video workflow and are exposed through a responsive local web interface. The custom SVM satisfies the training requirement, the end-to-end evaluation tests robustness rather than relying only on crop accuracy, and the measured performance is comfortably above the real-time threshold. The most important lesson was that complete-system evaluation can reveal failures hidden by excellent component metrics.

Future work should collect consented real images and videos, annotate objects with bounding boxes, compare a fine-tuned detector with the current proposal-plus-classifier approach, calibrate depth, quantify false negatives by hazard severity and test audio/haptic guidance with target users. A mobile deployment would also need model compression, battery profiling, offline inference and privacy-by-design review.

# References

Bradski, G. (2000). The OpenCV Library. Dr. Dobb's Journal of Software Tools.

Canny, J. (1986). A computational approach to edge detection. IEEE Transactions on Pattern Analysis and Machine Intelligence, 8(6), 679-698. https://doi.org/10.1109/TPAMI.1986.4767851

Cortes, C., & Vapnik, V. (1995). Support-vector networks. Machine Learning, 20, 273-297. https://doi.org/10.1007/BF00994018

Dalal, N., & Triggs, B. (2005). Histograms of oriented gradients for human detection. Proceedings of CVPR, 886-893. https://doi.org/10.1109/CVPR.2005.177

Pedregosa, F., et al. (2011). Scikit-learn: Machine learning in Python. Journal of Machine Learning Research, 12, 2825-2830.

Zivkovic, Z. (2004). Improved adaptive Gaussian mixture model for background subtraction. Proceedings of ICPR, 28-31. https://doi.org/10.1109/ICPR.2004.1333992

OpenCV. (2026). OpenCV documentation. https://docs.opencv.org/4.x/

Scikit-learn developers. (2026). SVC documentation. https://scikit-learn.org/stable/modules/generated/sklearn.svm.SVC.html

Gradio. (2026). Gradio documentation. https://www.gradio.app/docs

University of Wollongong in Dubai. (2026). CSCI435 project description, Spring 2026.

# Appendix A - Rubric traceability

| Criterion | Marks | Evidence |
| --- | --- | --- |
| Four or more tasks | 15 | Six integrated capabilities in VisionSystem; Figures 1 and 6. |
| Correctness and robustness | 15 | Seven conditions, edge-case assertions and class-aware IoU evaluation; Table 2. |
| UI and UX | 10 | Verified Gradio image/webcam and video tabs; Figure 2. |
| Model selection and training | 10 | 560 custom images, three-model comparison and saved SVM; Table 1. |
| Architecture and code quality | 10 | Typed functions/classes, unified notebook, README and architecture diagram. |
| Performance | 5 | 50.1 FPS measured versus 10 FPS requirement; Table 3. |
| Report quality | 15 | Required sections, diagrams, quantitative results, qualitative examples and critical analysis. |
| Live demonstration | 15 | Local interface, stable generated examples, 2:05 backup video and defence guide. |
| Teamwork | 5 | Named coding, documentation and defence responsibilities; Table 4. |

# Appendix B - Reproduction and defence checklist

Create a Python 3.12 virtual environment and install requirements.txt.

Open CSCI435_Project.ipynb and run all cells from top to bottom.

Confirm that the model comparison, confusion matrix, robustness table and performance assertion appear.

Open the printed local Gradio URL and process the generated defence image and sample video.

Show image enhancement, edge overlay, marker boxes, guidance, motion IDs and measured FPS.

Keep artifacts/CSCI435_Demonstration_Video.mp4 available as the required video and technical-failure backup.

Push the repository to GitHub or GitLab, grant the lecturer access and place the repository link in the Moodle submission.